# Work Instruction: Momentary Switch Setup

# Purpose

The purpose of this document is to provide instructions for initially creating and installing a momentary switch for the purpose of turning on the Raspberry Pi (RPi) Wi-Fi and FTP server for a predetermined period of time.

# Pre-requisites

The initial setup of the momentary switch will require:

- Momentary switch (like a standard, plastic doorbell)

- 25ft AUX cable

- Wire stripper

- Household tools

- x2 DuPont Wire Connectors

- Raspberry Pi Zero w/ Headers

# Instructions

| Step # | Work Instruction #1: Birdalytics RPi Initial Setup |
| --- | --- |
| 1 | Remove the AUX cable outer sheathing, revealing several wires.<image will be added later> |
| 2 | Choose two wires on both sides of the cable to strip the outer plastic from, revealing the bare metal wire using the wire strippers. .<image will be added later> Note: Color and number of wires not being used does not matter. |
| 3 | Using the household tools, loosen the screws on the momentary switch. Wrap the bare metal wire around and tighten the screw for a secure connection. Note: Overtightening will tear the metal, especially during movement. .<image will be added later> |
| 4 | Optional: To test the connection, a multimeter can be used to ensure there is positive continuity when the momentary switch is pressed. Place the multimeter into continuity mode, touch the probes to the other end of the exposed wires, press the switch, and the multimeter should indicate continuity. If not, clean/secure the connection. |
| 5 | Crimp the two DuPont wire connectors to the other ends of the wires. .<image will be added later> |
| 6 | Holding the Raspberry Pi in a vertical orientation with the GPIO pins on the right side, the pins are counted in numerical order. 1 and 2, 3 and 4, 5 and 6, etc. Place the DuPont wire connectors into pin 6 and pin 11. Color and order does not matter. Note: Pin 6 can be substituted for any ground pin, but pin 11 is predetermined within the code. .<image will be added later> |
| 7 | Optional: Work Instruction #1: RPi Initial Setup can be used to test the functionality of the momentary switch. |

# Revision History

| Revision | Description of Change | Effective Date |
| --- | --- | --- |
| 1 | Initial document | 1/1/2026 |