# Work Instruction: Birdalytics RPi Initial Setup

# Purpose

The purpose of this document is to provide instructions for initially setting up the Raspberry Pi (RPi) Zero W system, focusing on:

- Operating system installation & configuration

- Momentary switch configuration

- Wi-Fi hotspot setup

- File Transfer Protocol (FTP) server setup

Note: See Work Instruction (WI) #2 for setting up the RPi camera and associated camera software.

# Pre-requisites

The initial setup of the RPi will require:

- Raspberry Pi Zero W with plug-in Micro USB-B power

- Mini-HDMI to HDMI adapter, HDMI cable, and external monitor

- 32GB SD card

- Micro USB-B to USB-A adapter and USB-A keyboard

- Functional and installed momentary switch (see WI #3).

# Instructions

| Step # | Work Instruction #1: Birdalytics RPi Initial Setup |
| --- | --- |
| 1 | Flashing the 32GB SD card |
| 1.1 | Insert the SD card into a computer |
| 1.2 | Install and open Raspberry Pi Imager |
| 1.3 | Select your Raspberry Pi device: Raspberry Pi Zero |
| 1.4 | Choose operating system: Raspberry Pi OS (other) -> Raspberry Pi OS Lite (32-bit) |
| 1.5 | Select your storage device: 32GB SD card |
| 1.6 | Hostname: birdalytics-box{box number} Example for box #4, “birdalytics-box4” |
| 1.7 | Localization: Capital city -> Washington, D.C. Time zone -> America/New_York Keyboard layout -> us |
| 1.8 | User: Username -> birdalytics Password -> birdalytics123 |
| 1.9 | Wi-Fi: Type the SSID and password for an internet-accessible Wi-Fi for setup only |
| 1.10 | Disable SSH and Raspberry Pi Connect |
| 1.11 | Write to the SD card |
| 2 | Booting into the RPi |
| 2.1 | Insert the SD card into the RPi SD card slot |
| 2.2 | Connect a monitor (using the Mini HDMI adapter and HDMI cable) into the Mini HDMI port, keyboard (using the Micro USB-B to USB-A adapter) into the USB port, and a functional momentary switch (see WI #3). |
| 2.3 | Power the RPi using a stable power source into the PWR IN port. |
| 2.4 | First boot will take several minutes. On initialization, it will ask for username and password. Enter the credentials from step 1.8. |
| 3 | Removing the login screen |
| 3.1 | Type sudo raspi-config |
| 3.2 | Navigate to System Options -> Auto Login Note: Navigate using keyboard arrow keys and the Enter key |
| 3.3 | Select <Yes> for console auto login |
| 3.4 | Select <Ok> and <Finish> |
| 4 | Installing and configuring services |
| 4.1 | Ensure all packages are up to date, type: sudo apt update |
| 4.2 | Install required packages, type: sudo apt install -y python3 hostapd dnsmasq vsftpd |
| 4.3 | Disable auto-start of packages at boot, type: sudo systemcyl disable hostapd dnsmasq vsftpd |
| 4.4 | Configure the hostapd.conf, type: sudo nano /etc/hostapd/hostapd.conf and replace all contents with Appendix A. |
| 4.5 | Configure the dnsmasq.conf, type: sudo nano /etc/dnsmasq.conf and replace all contents with Appendix B. |
| 4.6 | Configure the vsftpd.conf, type: sudo nano /etc/vsftpd.conf and replace all contents with Appendix C. |
| 5 | Configuring the switch-activated Wi-Fi and FTP |
| 5.1 | Create the Python script that will start the Wi-Fi and FTP, type: sudo nano /home/pi/start_wifi_ftp.py and insert all contents from Appendix D |
| 5.2 | Create the Python script that will stop the Wi-Fi and FTP, type: sudo nano /home/pi/stop_wifi_ftp.py and insert all contents from Appendix E |
| 5.3 | Create the doorbell Python script, type: sudo nano /home/pi/doorbell.py and insert all contents from Appendix F |
| 5.4 | Create the doorbell service, type: sudo nano/etc/systemd/system/doorbell.service and insert all contents from Appendix G |
| 5.5 | Enable and start this doorbell service, type: sudo systemctl daemon-reload sudo systemctl enable doorbell.service sudo systemctl start doorbell.service |
| 6 | Installing and configuring an FTP mobile application |
| 6.1 | Using a smart phone, install FTPManager for iOS or CX File Manager for Android. Note: While many applications can perform FTP functions, these two apps are known for being free, highly rated, and user-friendly. Other apps are loaded with ads or paid features that are completely unnecessary. |
| 6.2 | While interfaces will vary based on the app, each one will have the basic “FTP Connection”. Create a new FTP connection. Note: These RPi’s are not configured for SFTP – use FTP. |
| 6.3 | Configure the FTP connection: Host Name/IP -> 192.168.50.1 Port -> 21 Username -> The RPi username configured in step 1.8 Password -> The RPi password configured in step 1.8 Note: You will NOT need to create a new connection for each birdbox. Only one connection/profile needs to be configured. This FTP connection is not expected to connect yet, see step 7. |
| 7 | Testing operations |
| 7.1 | Perform a fresh reboot of the RPi, type: sudo reboot Note: You should expect the RPi to boot and autologin to the terminal. If autologin fails, review step 3. |
| 7.2 | Ensure services are disabled on startup, type: sudo systemctl status dnsmasq vsftpd hostapd Note: You should expect that the “Active:” status shows dead. None of these services should be running. If they are, review step 4.3. |
| 7.3 | Ensure the momentary switch activates the Wi-Fi hotspot. Press the switch and wait for about 1 minute. Note: You should expect that a “Birdalytics” Wi-Fi network is available in your phone’s Wi-Fi list and/or you can reuse the status command from above to check that the services are now active and running. |
| 7.4 | Ensure the momentary switch activates the FTP server. If not done already, press the switch and wait for about 1 minute. Open the FTP mobile app and open the connection that was configured in step 6.3. Note: You should expect the app to open/show folders and files on the RPi, likely the stop, start, and doorbell scripts. Photos will be accessible from this screen. If the connection fails, troubleshoot using the status command from above, ensure the .conf files were edited/saved, and ensure the mobile app was configured correctly. |
| 7.5 | Assess the Wi-Fi duration. After 30 minutes, the Wi-Fi hotspot is expected to turn itself off to preserve battery life. Additionally, if the momentary button is pressed at any point during the 30 minutes, the timer will reset. Note: A log of when the momentary switch was pressed can be found in /home/pi/doorbell.log |

# Revision History

| Revision | Description of Change | Effective Date |
| --- | --- | --- |
| 1 | Initial document | 1/1/2026 |

# Appendix A: hostapd.conf

interface=wlan0 driver=nl80211 ssid=Birdalytics hw_mode=g channel=6 auth_algs=1 wpa=2 wpa_passphrase=birdbox123 wpa_key_mgmt=WPA-PSK rsn_pairwise=CCMP

# Appendix B: dnsmasq.conf

interface=wlan0 dhcp-range=192.168.50.2,192.168.50.10,255.255.255.0,24h

# Appendix C: vsftpd.conf

listen=YES listen_ipv6=NO anonymous_enable=NO local_enable=YES write_enable=YES chroot_local_user=YES allow_writeable_chroot=YES secure_chroot_dir=/var/run/vsftpd/empty

# Appendix D: start_wifi_ftp.sh

#!/bin/bash sudo ip addr add 192.168.50.1/24 dev wlan0 sudo ip link set wlan0 up sudo systemctl start hostapd sudo systemctl start dnsmasq sudo systemctl start vsftpd

# Appendix E: stop_wifi_ftp.sh

#!/bin/bash sudo systemctl stop vsftpd sudo systemctl stop hostapd sudo systemctl stop dnsmasq sudo ip link set wlan0 down sudo ip addr flush dev wlan0

# Appendix F: doorbell.py

import RPi.GPIO as GPIO import subprocess import threading import time import os DOORBELL_PIN = 17 TIMER_DURATION = 1800 hotspot_timer = None hotspot_running = False START_SCRIPT = '/home/pi/start_wifi_ftp.sh' STOP_SCRIPT = '/home/pi/stop_wifi_ftp.sh' LOG_FILE = '/home/pi/doorbell.log' def log_press(): timestamp = time.strftime('%Y-%m-%d %H:%M:%S') with open(LOG_FILE, "a") as f: f.write(f"{timestamp} - Doorbell pressed\n") subprocess.run(['logger', '-t', 'doorbell', f'Doorbell pressed at {timestamp}']) def stop_hotspot(): global hotspot_running subprocess.run(['sudo', STOP_SCRIPT]) hotspot_running = False def turn_on_wifi(channel): global hotspot_timer, hotspot_running log_press() if not hotspot_running: subprocess.run(['sudo', START_SCRIPT]) hotspot_running = True else: pass if hotspot_timer is not None: hotspot_timer.cancel() hotspot_timer = threading.Timer(TIMER_DURATION, stop_hotspot) hotspot_timer.start() GPIO.setmode(GPIO.BCM) GPIO.setup(DOORBELL_PIN, GPIO.IN, pull_up_down=GPIO.PUD_UP) GPIO.add_event_detect(DOORBELL_PIN, GPIO.FALLING, callback=turn_on_wifi, bouncetime=300) try: while True: time.sleep(1) except KeyboardInterrupt: GPIO.cleanup()

# Appendix G: doorbell.service

[Unit] Description=Doorbell listener After=multi-user.target [Service] Type=simple ExecStart=/usr/bin/python3 /home/pi/doorbell.py Restart=on-failure User=root WorkingDirectory=/home/pi [Install] WantedBy=multi-user.target